TUG:  CHARLES HUGHES

-Type of vessel:  Push Tug

-In survey to:  Lakes, Bays and Sounds

-Full Sub-Chapter M Compliant

-USCG Inspected (Certificate of Inspection/ Certificate of Documentation, December 2021)

Pilot House: Height of Eye @ 35’

Home port:  Baltimore, Maryland

Owner and Operator:  Vane Brothers

Designer:  Chesapeake Shipbuilding Shipbuilders and Naval Architects

Builder:  Chesapeake Shipbuilding (Salisbury, MD)

Delivered: January 2022

Hull construction material:  Steel

Superstructure construction material:  Steel

Deck construction material:  Steel

Length overall:  94’-0”

Beam:  34’-0”

Designed Draft:  8’-6”

Depth:  10’-6”

Main engine/s: (2) Caterpillar 3516 - 1,500 HP (each) @ 1600 RPM

Total horsepower: 3,000 horsepower

Speed:  10.5 kts

Reduction Gears:  Two (2) Twin Disc MGX 5600 6:1

Speed and Reduction Controls:  Twin Disc Controls

Propulsion:  Fixed Props…Hung Shen SS (5 blade)

Shaft Seals:  Simplan Dripless Shaft Seals (Simplex America)

Auxiliary engine/s:  Two (2) John Deere 4045)

Generator/s:  Marathon 99kw

Winch Engine:  n/a

Push Winches:  Patterson 65T Low Profile (with 1 ¾” Kevlar line)

Capstans; Two (2) Jon-Rie electric (fore and aft)

Steering system:  Hydraulic (Jastram Steering System)

Maximum speed:  10.5 kts. (lightship)

Cruising speed: 10 kts. (full load)

Fuel Capacity:  40k gallons

Potable Water:  5k gallons

Zero Discharge Tank Capacity:  4,800 gallons

Electronics:  Simrad and Furuno (installed by Rhodes Electronics, Houma, La.)

Radars: Simrad Halo Radars, (2) Simrad 3D, Touch Screen Monitors,Dual Rose Point electronic charting systems.

Depth sounder/s:  Simrad

Radios:  Icom

Autopilots:  Simrad AP70

Compass/es:  Simrad Gyro Compass, Simrad SAT Compass, Ritchie Constellation Magnetic compasses

Weatherfax:  Sirius Satellite Weather Receiver

GPS:  Simrad

Plotters:  Simrad

A.I.S.  Simrad

Audio visual system:  Color Marine Cameras (Splash Com)

Other electronics:  Intel Sat. Television

Other equipment installed:

-Grid Coolers: Fernstrum Box coolers

-Simplex Dripless Shaft Seals

-A-Head (12 man) MSD System (4k gallon Zero Discharge Tank)

-Kaydon Fuel Oil Cleaner

-Hartzell Supply and Exhaust fans

-Carlisle & Finch Remote Operating Spot Lights

-LED Lighting

-Alarm system:  Ships Alarm Panel (Unlimited Controls & Supply, Houma, La.)

-Paints/coatings:  International

-Windows:  Bowmar(heated)

-Doors and Hatches: Freeman

-Carrier heating and A/C  (Custom Installation/ Multi-Marine Services)

-Quincy 325 Air Compressors (Two (2))

-Kim Hot Start (block heaters)

-Fendering:  M&M Bumpers (Houma, La.)

-Flooring/Deck surface finishes:  Dex-O Tex  Poured Flooring

-Interior fit out/furnishings:  Fit out by Trident Ships Carpenters/ Furnishings by Industrial Alliance

-Safety equipment: (Extensive/ Vane Brothers Marine Safety)

-Liferaft/s:  RFD (10 Man) Vane Brothers Marine Safety

-Hiller C02 Fire Suppression System (C02)

Crew: Berthing for seven (7) – operational crew is generally 5 (captain, mate, engineer, two deckhands)

Operational area:  Northeast United States