Blount Hull #MDNR-373

94’x27’- Buoy Tender/Ice Breaker

Principal Particulars

Length, Overall = 94’-0”

Beam = 27’-0”

Depth, Molded = 10’-0”

Mean Draft =	 4’-6”

Builder:	           	                             Blount Boats, Inc. Warren RI

Naval Architect/Designer;               BMT Designers & Planners Inc.

Owner:                                             Maryland Department of Natural Resources

Owners Representative:                   Gryphon Technologies (A ManTech Company)

Owner/Operator: 			Maryland Department of Natural Resources

Crew:	 		              6

Passengers:                                       None

Speed:				12 knots Max, (10 knots cruising loaded @ 1650 RPM).

Tank Capacities:

Fuel Oil:	1987 Gallons

Fresh Water:                      1000 Gallons

Sewage:                             1000 Gallons Black – 500 Gallons Gray

Hull construction: 	              Steel (CNC Lofting – Boksa Marine)

Hatches & Manholes:                      Freeman Marine & NABRICO

Mooring:                                          NABRICO

Coatings:			International (SurfaceWorx), Paint Sub-contractor R.J. Forbes

Main propulsion:			(2) Cummins QSK19, Tier III, 750HP@1800RPM, Heavy Duty Rating; (Cummins Northeast)

Marine Gear:			(2) Twin Disc MGX-5202SC 2.48:1 Ratio North Atlantic Power Products

Propellers:			(2) 42X27 – 5 Blade NIBRAL, Michigan Wheel from New England Propeller

Shafts:				(2) 3.75” Diameter Aquamet 17 from NSA and machined By H&H Propeller, Salem, MA.

Shaft Bearings:                                 Duramax

Shaft Seals:			Duramax Dry-Seal

Engine Controls:			Twin Disc EC300 (3 stations)

Rudders:                                           Blount Built

Ship’s Service Power:		(2) 55kW, Cummins Onan

Switchboard:                                    Power Panels Inc., Morgan City LA.

Engine Cooling:			Welded Channel Iron for Mains / WEKA Box Coolers for Gens

Exhaust System;                               MAXIM 8 inch & 4 inch

Steering System:		Jastrom B2-76-400-1-35 (3 Station) (RG Group, York PA)

Fire Pump:	Flowmax 8, 2x2

Bilge Pump:	Flowmax 8, 2x2

Sewage Pump:                                  Tortuga Diaphragm TOR-3P60

Engine Room Ventilation:               Delta T (2) 3HP 21” Intake

Lovers & Dampers:                          Delta T (2) Alum Mist Eliminators

HVAC System:                               (1) 160,000 BTU Burnham Oil Fired Boiler, Modine Hydronic Heaters & Base Board

HVAC System:                                Pilot House & Crew Cabin & Galley, Fujitsu Split Duct

Window Defrosters:                        Heater Craft

Fire Suppression:	              KIDDE (2) 100# CO2 Fire Suppression System (Hiller Systems Inc)

Engine Room Insulation:                 Anchor Insulation Co, A-60 / 2-3” Mineral Wool, 1” Acoustic Navy Hull Board Finish.

Doors:		Beclawat

Windows:		Beclawat & NABRICO Port Lights

Ceiling:                                            Armstrong

Joinery:                                            Isonell (Pole Star Marine/Offshore)

Crew & Galley Furniture:               Pole Star Marine/Offshore

Flooring:                                          Lonseal (Karpet Klinic Cranston RI)

Lighting:                                          L.C. Doan

TV & CCTV:                                  Cay Electronics

Fire Detection:                                Firex-Fireboy

PA & Communications:                  David Clark Wireless Headsets

Alarms:                                            EMI Industries, LLC, Master Panel (ER) & Slave Panel (PH).

TLI:                                                  EMI Industries

Search Lights:       		(2) Carlisle & Finch 12” Searchlight With Remote Control & Wide Beam Lights

Compass:			Ritchie B-463 Steelboat Compass

Compass Adjuster; Cape Compass, West Falmouth MA.

Nav Equip; Cay Electronics:           Cay Electronics, Portsmouth RI

Lifesaving:			Fire Extinguishers (Firex Inc. Portsmouth RI)

(26 Adult) Imerial 160RT

(1) 180# Fortress Anchor

Certification:

- Tonnage; ABS

- FCC; Chris Electronics

- Stability Test; Mr. Chris Melo, Naval Architect